Instruction for Serial Download

To upgrade the Software of your IRD, you need to prepare as follows

1.Personal Computer.

2.Serial Cable (Cross Type)

3.*.EXE (Executable)

4.Download File(*.tdf)

Personal Computer

Your Computer should have a COM1 Port in it.

Serial Cable Connection Diagram

PC Serial Port ( Female, 9 Pin)        IRD Modem Port ( Female, 9 Pin)

Step 1.

Connect modem port or data port to the serial cable between PC and IRD

Step 2.

Disconnect Power plug from IRD.

Step 3.

Execute Down Loader.EXE file in WINDOWS mode by double clicking the Loader.EXE file and there will be a pop-up screen

Step 4

In the pop-up screen, please select COM 1 as the COM port and then click on “Download Start” and there will be a pop-up menu to select the software to be downloaded to the receiver which is

“ *.tdf ”.

Baud_rate

| Value | Speed |
| --- | --- |
| 0 | 115200 |
| 1 | 57600 |
| 2 | 38400 |
| 3 | 19200 |
| 4 | 9600 |

[Example]

Step 5.

Connect Power Plug in to IRD and power on the IRD. And then on the pop-up menu on the computer there will be a status of how many percent the software is being downloaded to the receiver.

Step 6.

Wait for about 3 to 4 minutes depending on your computer.

Once the message of "End" is displayed on front panel of IRD. Then you can close the download  software tool and power off your receiver.