Instruction for Serial Download

To upgrade the Software of your IRD , you need to prepare as follows:-

1. Personal Computer.

2. Serial Cable (Cross Type)

3. mon56.EXE (Executable)

4. Download File(MEDIA214.hex)

Personal Computer:-

Your Computer should have a COM1 Port in it.

Serial Cable Connection Diagram

PC Serial Port ( Female, 9 Pin)        IRD Modem Port ( Female, 9 Pin)

Step 1.

Connect modem port or data port to the serial cable between PC and IRD

Step 2.

Disconnect Power plug from IRD.

Step 3.

Double Click on mon56k.exe file in WINDOWS mode

Step 4

Then proceed to click on the File Open and select the software file “MEDIA214.hex”

Step 5

Once this is done proceed to click on the Start button.

Step 6.

Then connect Power Plug in to IRD and switch the receiver on. Once the receiver is switched on the software will automatically be downloaded from the PC to the receiver.

Step 7.

Waiting for about 1 minutes.

Message of "End" is displayed on front panel of IRD and also the message “Transfer Completed” will be displayed on the computer.